Captura Ethernet de la aplicación TFTP con el Cisco 2600

Record #9       (From Hub To Node) Captured on 03.24.06 at 19:41:49.861345797 Length =    66

------------  ETHER Header  ------------

ETHER: Destination: Edimax-37-E6-53 (00-00-B4-37-E6-53)

ETHER: Source: Cisco-09-A1-44 (00-00-0C-09-A1-44)

ETHER: Protocol: IP

ETHER: FCS: 2B4CCA13

------------  IP Header  ------------

IP: Version = 4

IP: Header length = 20

IP: Type of service = 0

IP:     000. .... Precedence = Routine(0)

IP:     ...0 .... Delay = Normal (0)

IP:     .... 0... Throughput = Normal (0)

IP:     .... .0.. Reliability = Normal (0)

IP: Packet length = 48

IP: Id = 0

IP: Fragmentation Info = 0x0000

IP:     .0.. ....  .... .... Don't Fragment Bit = FALSE

IP:     ..0. ....  .... .... More Fragments Bit = FALSE

IP:     ...0 0000  0000 0000 Fragment offset = 0

IP: Time to live = 254

IP: Protocol = UDP (17)

IP: Header checksum = 4F77

IP: Source address = 192.168.200.50

IP: Destination address = 128.100.100.6

------------  UDP Header  ------------

UDP: Source port = 9647

UDP: Destination port = tftp (69)

UDP: Length = 28

UDP: Checksum = 124B

------------  TFTP Header  ------------

TFTP: Opcode = Write request (2)

TFTP: Filename = c2600-confg

TFTP: Mode = octet

Record #10      (From Hub To Node) Captured on 03.24.06 at 19:41:49.895814697 Length =    64

------------  ETHER Header  ------------

ETHER: Destination: Cisco-09-A1-44 (00-00-0C-09-A1-44)

ETHER: Source: Edimax-37-E6-53 (00-00-B4-37-E6-53)

ETHER: Protocol: IP

ETHER: FCS: B7698AC7

------------  IP Header  ------------

IP: Version = 4

IP: Header length = 20

IP: Type of service = 0

IP:     000. .... Precedence = Routine(0)

IP:     ...0 .... Delay = Normal (0)

IP:     .... 0... Throughput = Normal (0)

IP:     .... .0.. Reliability = Normal (0)

IP: Packet length = 32

IP: Id = 96

IP: Fragmentation Info = 0x0000

IP:     .0.. ....  .... .... Don't Fragment Bit = FALSE

IP:     ..0. ....  .... .... More Fragments Bit = FALSE

IP:     ...0 0000  0000 0000 Fragment offset = 0

IP: Time to live = 64

IP: Protocol = UDP (17)

IP: Header checksum = 0CF2

IP: Source address = 128.100.100.6

IP: Destination address = 192.168.200.50

------------  UDP Header  ------------

UDP: Source port = 1029

UDP: Destination port = 9647

UDP: Length = 12

UDP: Checksum = 68D8

------------  TFTP Header  ------------

TFTP: Opcode = Ack  (4)

TFTP: Block number = 0

Record #11      (From Hub To Node) Captured on 03.24.06 at 19:41:50.367170799 Length =   562

------------  ETHER Header  ------------

ETHER: Destination: Edimax-37-E6-53 (00-00-B4-37-E6-53)

ETHER: Source: Cisco-09-A1-44 (00-00-0C-09-A1-44)

ETHER: Protocol: IP

ETHER: FCS: CDB5983C

------------  IP Header  ------------

IP: Version = 4

IP: Header length = 20

IP: Type of service = 0

IP:     000. .... Precedence = Routine(0)

IP:     ...0 .... Delay = Normal (0)

IP:     .... 0... Throughput = Normal (0)

IP:     .... .0.. Reliability = Normal (0)

IP: Packet length = 544

IP: Id = 1

IP: Fragmentation Info = 0x0000

IP:     .0.. ....  .... .... Don't Fragment Bit = FALSE

IP:     ..0. ....  .... .... More Fragments Bit = FALSE

IP:     ...0 0000  0000 0000 Fragment offset = 0

IP: Time to live = 254

IP: Protocol = UDP (17)

IP: Header checksum = 4D86

IP: Source address = 192.168.200.50

IP: Destination address = 128.100.100.6

------------  UDP Header  ------------

UDP: Source port = 9647

UDP: Destination port = 1029

UDP: Length = 524

UDP: Checksum = 589E

------------  TFTP Header  ------------

TFTP: Opcode = Data (3)

TFTP: Block number = 1

TFTP: 512 bytes of data

Record #12      (From Hub To Node) Captured on 03.24.06 at 19:41:50.368224699 Length =    64

------------  ETHER Header  ------------

ETHER: Destination: Cisco-09-A1-44 (00-00-0C-09-A1-44)

ETHER: Source: Edimax-37-E6-53 (00-00-B4-37-E6-53)

ETHER: Protocol: IP

ETHER: FCS: 1739CFEE

------------  IP Header  ------------

IP: Version = 4

IP: Header length = 20

IP: Type of service = 0

IP:     000. .... Precedence = Routine(0)

IP:     ...0 .... Delay = Normal (0)

IP:     .... 0... Throughput = Normal (0)

IP:     .... .0.. Reliability = Normal (0)

IP: Packet length = 32

IP: Id = 97

IP: Fragmentation Info = 0x0000

IP:     .0.. ....  .... .... Don't Fragment Bit = FALSE

IP:     ..0. ....  .... .... More Fragments Bit = FALSE

IP:     ...0 0000  0000 0000 Fragment offset = 0

IP: Time to live = 64

IP: Protocol = UDP (17)

IP: Header checksum = 0CF1

IP: Source address = 128.100.100.6

IP: Destination address = 192.168.200.50

------------  UDP Header  ------------

UDP: Source port = 1029

UDP: Destination port = 9647

UDP: Length = 12

UDP: Checksum = 68D7

------------  TFTP Header  ------------

TFTP: Opcode = Ack  (4)

TFTP: Block number = 1

Record #13      (From Hub To Node) Captured on 03.24.06 at 19:41:50.868583497 Length =   562

------------  ETHER Header  ------------

ETHER: Destination: Edimax-37-E6-53 (00-00-B4-37-E6-53)

ETHER: Source: Cisco-09-A1-44 (00-00-0C-09-A1-44)

ETHER: Protocol: IP

ETHER: FCS: 2EC02A10

------------  IP Header  ------------

IP: Version = 4

IP: Header length = 20

IP: Type of service = 0

IP:     000. .... Precedence = Routine(0)

IP:     ...0 .... Delay = Normal (0)

IP:     .... 0... Throughput = Normal (0)

IP:     .... .0.. Reliability = Normal (0)

IP: Packet length = 544

IP: Id = 2

IP: Fragmentation Info = 0x0000

IP:     .0.. ....  .... .... Don't Fragment Bit = FALSE

IP:     ..0. ....  .... .... More Fragments Bit = FALSE

IP:     ...0 0000  0000 0000 Fragment offset = 0

IP: Time to live = 254

IP: Protocol = UDP (17)

IP: Header checksum = 4D85

IP: Source address = 192.168.200.50

IP: Destination address = 128.100.100.6

------------  UDP Header  ------------

UDP: Source port = 9647

UDP: Destination port = 1029

UDP: Length = 524

UDP: Checksum = 01FC

------------  TFTP Header  ------------

TFTP: Opcode = Data (3)

TFTP: Block number = 2

TFTP: 512 bytes of data

Record #14      (From Hub To Node) Captured on 03.24.06 at 19:41:50.869651997 Length =    64

------------  ETHER Header  ------------

ETHER: Destination: Cisco-09-A1-44 (00-00-0C-09-A1-44)

ETHER: Source: Edimax-37-E6-53 (00-00-B4-37-E6-53)

ETHER: Protocol: IP

ETHER: FCS: 5D6FE5A1

------------  IP Header  ------------

IP: Version = 4

IP: Header length = 20

IP: Type of service = 0

IP:     000. .... Precedence = Routine(0)

IP:     ...0 .... Delay = Normal (0)

IP:     .... 0... Throughput = Normal (0)

IP:     .... .0.. Reliability = Normal (0)

IP: Packet length = 32

IP: Id = 98

IP: Fragmentation Info = 0x0000

IP:     .0.. ....  .... .... Don't Fragment Bit = FALSE

IP:     ..0. ....  .... .... More Fragments Bit = FALSE

IP:     ...0 0000  0000 0000 Fragment offset = 0

IP: Time to live = 64

IP: Protocol = UDP (17)

IP: Header checksum = 0CF0

IP: Source address = 128.100.100.6

IP: Destination address = 192.168.200.50

------------  UDP Header  ------------

UDP: Source port = 1029

UDP: Destination port = 9647

UDP: Length = 12

UDP: Checksum = 68D6

------------  TFTP Header  ------------

TFTP: Opcode = Ack  (4)

TFTP: Block number = 2

Record #15      (From Hub To Node) Captured on 03.24.06 at 19:41:51.128120199 Length =   142

------------  ETHER Header  ------------

ETHER: Destination: Edimax-37-E6-53 (00-00-B4-37-E6-53)

ETHER: Source: Cisco-09-A1-44 (00-00-0C-09-A1-44)

ETHER: Protocol: IP

ETHER: FCS: 2D11C453

------------  IP Header  ------------

IP: Version = 4

IP: Header length = 20

IP: Type of service = 0

IP:     000. .... Precedence = Routine(0)

IP:     ...0 .... Delay = Normal (0)

IP:     .... 0... Throughput = Normal (0)

IP:     .... .0.. Reliability = Normal (0)

IP: Packet length = 124

IP: Id = 3

IP: Fragmentation Info = 0x0000

IP:     .0.. ....  .... .... Don't Fragment Bit = FALSE

IP:     ..0. ....  .... .... More Fragments Bit = FALSE

IP:     ...0 0000  0000 0000 Fragment offset = 0

IP: Time to live = 254

IP: Protocol = UDP (17)

IP: Header checksum = 4F28

IP: Source address = 192.168.200.50

IP: Destination address = 128.100.100.6

------------  UDP Header  ------------

UDP: Source port = 9647

UDP: Destination port = 1029

UDP: Length = 104

UDP: Checksum = 56BE

------------  TFTP Header  ------------

TFTP: Opcode = Data (3)

TFTP: Block number = 3

TFTP: 92 bytes of data

Record #16      (From Hub To Node) Captured on 03.24.06 at 19:41:51.128659899 Length =    64

------------  ETHER Header  ------------

ETHER: Destination: Cisco-09-A1-44 (00-00-0C-09-A1-44)

ETHER: Source: Edimax-37-E6-53 (00-00-B4-37-E6-53)

ETHER: Protocol: IP

ETHER: FCS: 43E40F4B

------------  IP Header  ------------

IP: Version = 4

IP: Header length = 20

IP: Type of service = 0

IP:     000. .... Precedence = Routine(0)

IP:     ...0 .... Delay = Normal (0)

IP:     .... 0... Throughput = Normal (0)

IP:     .... .0.. Reliability = Normal (0)

IP: Packet length = 32

IP: Id = 99

IP: Fragmentation Info = 0x0000

IP:     .0.. ....  .... .... Don't Fragment Bit = FALSE

IP:     ..0. ....  .... .... More Fragments Bit = FALSE

IP:     ...0 0000  0000 0000 Fragment offset = 0

IP: Time to live = 64

IP: Protocol = UDP (17)

IP: Header checksum = 0CEF

IP: Source address = 128.100.100.6

IP: Destination address = 192.168.200.50

------------  UDP Header  ------------

UDP: Source port = 1029

UDP: Destination port = 9647

UDP: Length = 12

UDP: Checksum = 68D5

------------  TFTP Header  ------------

TFTP: Opcode = Ack  (4)

TFTP: Block number = 3